KBLH-E61规格详细说明

版本说明

| 版本 | 日期 | 说明 | 备注 |
| --- | --- | --- | --- |
| V0.1 | 2020.06.22 | 创建并完成首次版本 |  |

KBLH-E61规格详细说明

KBLH-E61主板框架图

| KBLH-E61详细规格 |  |  |
| --- | --- | --- |
| System |  |  |
| Form Factor | COM Express Basic size, Type 6 |
| CPU | Intel® Core™ 6/7th 45W Platform Processor with PCH HM175 |
| Memory Type | DDR4 SODIMM x 2 |
| Max.Memory Capacity | up to 32 GB |
| BIOS | AMI BIOS |
| Watchdog Timer | Yes |
| Power Requirement | Standard +12V |
| Power Consumption | - |
| Dimension | 4.92" x 3.75" (125 mm x 95 mm) |
| Operating Temperature | -4˚F ~ 158˚F (-20˚C ~ 70˚C) |
| Storage Temperature | -4˚F ~ 158˚F (-20˚C ~ 70˚C)° |
| Display |  |
| Display Controller | Intel® HD Graphics 630 |
| Video Output | LVDS or eDP, DDI x 3 |
| LVDS Interface | Supports 18-bit and 24-bit Single/Dual channel |
| I/O |  |
| Ethernet | Intel® I219, Gigabit Ethernet |
| Audio | HD Audio |
| USB port | USB2.0 x 8, USB3.2 Gen 2 x 4 |
| Serial Port | 2-Wire UART (Tx/Rx) x 2 |
| HDD Interface | SATA 3 x 4 |
| Expansion Slot | PCIe[x16] x 1 PCIe[x1] x 8 I2C LPC SMBus |
| GPIO | 4 x GPI 4 x GPO |

KBLH-E61主板框架图

主板布局图

接口PIN 定义

| A Role |  |  | B Role |  |  |
| --- | --- | --- | --- | --- | --- |
| Pin | Name | Description | Pin | Name | Description |
| A1 | GND (fixed) | GND (fixed) | B1 | GND (fixed) | GND (fixed) |
| A2 | GBE0_MDI3- | GBE0_MDI3- | B2 | GBE0_ACT# | 网络Active 指示灯，低电平有效 |
| A3 | GBE0_MDI3+ | GBE0_MDI3+ | B3 | LPC_FRAME# | LPC_FRAME# |
| A4 | GBE0_LINK100# | 100M LINK LED, 低电平有效 | B4 | LPC_AD0 | LPC_AD0 |
| A5 | GBE0_LINK1000# | 1000M LINK LED, 低电平有效 | B5 | LPC_AD1 | LPC_AD1 |
| A6 | GBE0_MDI2- | GBE0_MDI2- | B6 | LPC_AD2 | LPC_AD2 |
| A7 | GBE0_MDI2+ | GBE0_MDI2+ | B7 | LPC_AD3 | LPC_AD3 |
| A8 | GBE0_LINK# | 10M/100M/1000M LINK LED， 低电平有效 | B8 | LPC_DRQ0# | NC |
| A9 | GBE0_MDI1- | GBE0_MDI1- | B9 | LPC_DRQ1# | NC |
| A10 | GBE0_MDI1+ | GBE0_MDI1+ | B10 | LPC_CLK | LPC_CLK |
| A11 | GND (fixed) | GND (fixed) | B11 | GND (fixed) | GND (fixed) |
| A12 | GBE0_MDI0- | GBE0_MDI0- | B12 | PWRBTN# | 开关按键触发信号，INPUT，低电平脉冲触发有效 |
| A13 | GBE0_MDI0+ | GBE0_MDI0+ | B13 | SMB_CK | SMBUS CLK 信号，SUS电源 |
| A14 | GBE0_CTREF | GBE0_CTREF ，0.1uF电容直接接GND | B14 | SMB_DAT | SMBUS DAT 信号，SUS电源 |
| A15 | SUS_S3# | 电源管理信号，用于指示S3状态，output，低电平有效 | B15 | SMB_ALERT# | SMB_ALERT# |
| A16 | SATA0_TX+ | SATA0_TX+ | B16 | SATA1_TX+ | SATA1_TX+ |
| A17 | SATA0_TX- | SATA0_TX- | B17 | SATA1_TX- | SATA1_TX- |
| A18 | SUS_S4# | 电源管理信号，用于指示S4状态，output，低电平有效 | B18 | SUS_STAT# | Suspend指示信号，Output，用于通知LPC 设备 |
| A19 | SATA0_RX+ | SATA0_RX+ | B19 | SATA1_RX+ | SATA1_RX+ |
| A20 | SATA0_RX- | SATA0_RX- | B20 | SATA1_RX- | SATA1_RX- |
| A21 | GND (fixed) | GND (fixed) | B21 | GND (fixed) | GND (fixed) |
| A22 | SATA2_TX+ | SATA2_TX+ | B22 | SATA3_TX+ | SATA3_TX+ |
| A23 | SATA2_TX- | SATA2_TX- | B23 | SATA3_TX- | SATA3_TX- |
| A24 | SUS_S5# | 电源管理信号，用于指示S5状态，output，低电平有效 | B24 | PWR_OK | 底板PWROK信号，用于指示底板的器件供电指示，高电平有效，不用可以悬空。 |
| A25 | SATA2_RX+ | SATA2_RX+ | B25 | SATA3_RX+ | SATA3_RX+ |
| A26 | SATA2_RX- | SATA2_RX- | B26 | SATA3_RX- | SATA3_RX- |
| A27 | BATLOW# | 电池低电指示信号，INPUT，低电平有效 | B27 | WDT | WDT输出信号，低电平有效，不用可以悬空 |
| A28 | (S)ATA_ACT# | SATA LED指示灯信号，OUTPUT，低电平有效 | B28 | HDA_SDIN2 | HDA_SDIN2 |
| A29 | HDA_SYNC | HDA_SYNC | B29 | HDA_SDIN1 | HDA_SDIN1 |
| A30 | HDA_RST# | HDA_RST# | B30 | HDA_SDIN0 | HDA_SDIN0 |
| A31 | GND (fixed) | GND (fixed) | B31 | GND (fixed) | GND (fixed) |
| A32 | HDA_BITCLK | HDA_BITCLK | B32 | SPKR | 蜂鸣器Buzzer控制信号，Output，PWM信号 |
| A33 | HDA_SDOUT | HDA_SDOUT | B33 | I2C_CK | EC I2C CLK 信号，上拉到EC电源 |
| A34 | BIOS_DIS0# | Strap PIN，选择核心板BIOS还是载BIOS有效，Input，低电平有效 | B34 | I2C_DAT | EC I2C CLK 信号，上拉到EC电源 |
| A35 | THRMTRIP# | 过热检测信号，Output，低有效 | B35 | THRM# | 过热指示信号，Input，低电平有效，会使CPU降频 |
| A36 | USB6- | USB6- | B36 | USB7- | USB7- |
| A37 | USB6+ | USB6+ | B37 | USB7+ | USB7+ |
| A38 | USB_6_7_OC# | USB OC功能，Input，低电平有效 | B38 | USB_4_5_OC# | USB OC功能，Input，低电平有效 |
| A39 | USB4- | USB4- | B39 | USB5- | USB5- |
| A40 | USB4+ | USB4+ | B40 | USB5+ | USB5+ |
| A41 | GND (fixed) | GND (fixed) | B41 | GND (fixed) | GND (fixed) |
| A42 | USB2- | USB2- | B42 | USB3- | USB3- |
| A43 | USB2+ | USB2+ | B43 | USB3+ | USB3+ |
| A44 | USB_2_3_OC# | USB OC功能，Input，低电平有效 | B44 | USB_0_1_OC# | USB OC功能，Input，低电平有效 |
| A45 | USB0- | USB0- | B45 | USB1- | USB1- |
| A46 | USB0+ | USB0+ | B46 | USB1+ | USB1+ |
| A47 | VCC_RTC | RTC Battery 电源输入 | B47 | EXCD1_PERST# | EXCD1_PERST# |
| A48 | EXCD0_PERST# | EXCD0_PERST# | B48 | EXCD1_CPPE# | EXCD1_CPPE# |
| A49 | EXCD0_CPPE# | EXCD0_CPPE# | B49 | SYS_RESET# | 用于Reset系统，Input，低电平有效，不用可以悬空 |
| A50 | LPC_SERIRQ | LPC_SERIRQ | B50 | CB_RESET# | 用于系统Reset平台设备，Ouput，低电平有效，不用可以悬空 |
| A51 | GND (fixed) | GND (fixed) | B51 | GND (fixed) | GND (fixed) |
| A52 | PCIE_TX5+ | PCIE_TX5+ | B52 | PCIE_RX5+ | PCIE_RX5+ |
| A53 | PCIE_TX5- | PCIE_TX5- | B53 | PCIE_RX5- | PCIE_RX5- |
| A54 | GPI0 | GPI0 | B54 | GPO1 | GPO1 |
| A55 | PCIE_TX4+ | PCIE_TX4+ | B55 | PCIE_RX4+ | PCIE_RX4+ |
| A56 | PCIE_TX4- | PCIE_TX4- | B56 | PCIE_RX4- | PCIE_RX4- |
| A57 | GND | GND | B57 | GPO2 | GPO2 |
| A58 | PCIE_TX3+ | PCIE_TX3+ | B58 | PCIE_RX3+ | PCIE_RX3+ |
| A59 | PCIE_TX3- | PCIE_TX3- | B59 | PCIE_RX3- | PCIE_RX3- |
| A60 | GND (fixed) | GND (fixed) | B60 | GND (fixed) | GND (fixed) |
| A61 | PCIE_TX2+ | PCIE_TX2+ | B61 | PCIE_RX2+ | PCIE_RX2+ |
| A62 | PCIE_TX2- | PCIE_TX2- | B62 | PCIE_RX2- | PCIE_RX2- |
| A63 | GPI1 | GPI1 | B63 | GPO3 | GPO3 |
| A64 | PCIE_TX1+ | PCIE_TX1+ | B64 | PCIE_RX1+ | PCIE_RX1+ |
| A65 | PCIE_TX1- | PCIE_TX1- | B65 | PCIE_RX1- | PCIE_RX1- |
| A66 | GND | GND | B66 | WAKE0# | 用于S3，S4状态唤醒主机，Input，低脉冲触发 |
| A67 | GPI2 | GPI2 | B67 | WAKE1# | 用于S3，S4状态唤醒主机，Input，低脉冲触发 |
| A68 | PCIE_TX0+ | PCIE_TX0+ | B68 | PCIE_RX0+ | PCIE_RX0+ |
| A69 | PCIE_TX0- | PCIE_TX0- | B69 | PCIE_RX0- | PCIE_RX0- |
| A70 | GND (fixed) | GND (fixed) | B70 | GND (fixed) | GND (fixed) |
| A71 | LVDS_A0+ | LVDS_A0+ | B71 | LVDS_B0+ | LVDS_B0+ |
| A72 | LVDS_A0- | LVDS_A0- | B72 | LVDS_B0- | LVDS_B0- |
| A73 | LVDS_A1+ | LVDS_A1+ | B73 | LVDS_B1+ | LVDS_B1+ |
| A74 | LVDS_A1- | LVDS_A1- | B74 | LVDS_B1- | LVDS_B1- |
| A75 | LVDS_A2+ | LVDS_A2+ | B75 | LVDS_B2+ | LVDS_B2+ |
| A76 | LVDS_A2- | LVDS_A2- | B76 | LVDS_B2- | LVDS_B2- |
| A77 | LVDS_VDD_EN | LVDS 逻辑电源使能，Input，高电平有效 | B77 | LVDS_B3+ | LVDS_B3+ |
| A78 | LVDS_A3+ | LVDS_A3+ | B78 | LVDS_B3- | LVDS_B3- |
| A79 | LVDS_A3- | LVDS_A3- | B79 | LVDS_BKLT_EN | LVDS背光开启信号，Output，高有效 |
| A80 | GND (fixed) | GND (fixed) | B80 | GND (fixed) | GND (fixed) |
| A81 | LVDS_A_CK+ | LVDS_A_CK+ | B81 | LVDS_B_CK+ | LVDS_B_CK+ |
| A82 | LVDS_A_CK- | LVDS_A_CK- | B82 | LVDS_B_CK- | LVDS_B_CK- |
| A83 | LVDS_I2C_CK | LVDS_I2C_CK | B83 | LVDS_BKLT_CTRL | LVDS背光调节信号,Output，PWM信号 |
| A84 | LVDS_I2C_DAT | LVDS_I2C_DAT | B84 | VCC_5V_SBY | VCC_5V_SBY |
| A85 | GPI3 | GPI3，Input，默认高电平 | B85 | VCC_5V_SBY | VCC_5V_SBY |
| A86 | RSVD | NC | B86 | VCC_5V_SBY | VCC_5V_SBY |
| A87 | eDP-HPD | eDP-HPD信号，高有效 | B87 | VCC_5V_SBY | VCC_5V_SBY |
| A88 | PCIE_CLK+ | PCIE_CLK+ | B88 | BIOS_DIS1# | Strap PIN，选择核心板BIOS还是载BIOS有效，Input，低电平有效 |
| A89 | PCIE_CLK- | PCIE_CLK- | B89 | EC_PWM1 | EC GPIO信号，详细使用，见其他文档 |
| A90 | GND (fixed) | GND (fixed) | B90 | GND (fixed) | GND (fixed) |
| A91 | SPI_POWER | SPI_POWER， 3.3V电平 | B91 | EC_PWM6 | EC GPIO信号，详细使用，见其他文档 |
| A92 | SPI_MISO | SPI_MISO | B92 | EC_PWM7 | EC GPIO信号，详细使用，见其他文档 |
| A93 | GPO0 | GPO0 | B93 |  | NC |
| A94 | SPI_CLK | SPI_CLK | B94 |  | NC |
| A95 | SPI_MOSI | SPI_MOSI | B95 | BATT1_ECIN# | EC GPIO信号，详细使用，见其他文档 |
| A96 | TPM_PP | NC | B96 |  | NC |
| A97 | TYPE10# | NC | B97 | SPI_CS# | SPI_CS# |
| A98 | SER0_TX | 核心板Uart TX信号 | B98 | PCIE1-4_SATA_PCIE | PCIE1-4_SATA_PCIE |
| A99 | SER0_RX | 核心板Uart RX信号 | B99 | SATA0-1_SATA_PCIE | SATA0-1_SATA_PCIE |
| A101 | SER1_TX | 核心板Uart TX信号 | B101 | FAN_PWMOUT | 风扇PWM控制信号，Output |
| A102 | SER1_RX | 核心板Uart RX信号 | B102 | FAN_TACHIN | 风扇转速信号，Input |
| A103 | LID# | LID 信号输入，低电平有效 | B103 | SLEEP# | 睡眠控制信号，Input，低脉冲则触发进入睡眠动作。 |
| A104 | VCC_12V | VCC_12V | B104 | VCC_12V | VCC_12V |
| A105 | VCC_12V | VCC_12V | B105 | VCC_12V | VCC_12V |
| A106 | VCC_12V | VCC_12V | B106 | VCC_12V | VCC_12V |
| A107 | VCC_12V | VCC_12V | B107 | VCC_12V | VCC_12V |
| A108 | VCC_12V | VCC_12V | B108 | VCC_12V | VCC_12V |
| A109 | VCC_12V | VCC_12V | B109 | VCC_12V | VCC_12V |
| A110 | GND (fixed) | GND (fixed) | B110 | GND (fixed) | GND (fixed) |

| C Role |  |  | D Role |  |  |
| --- | --- | --- | --- | --- | --- |
| Pin | Name |  | Pin | Name |  |
| C1 | GND (fixed) | GND (fixed) | D1 | GND (fixed) | GND (fixed) |
| C2 | GND | GND | D2 | GND | GND |
| C3 | USB_SSRX0- | USB_SSRX0- | D3 | USB_SSTX0- | USB_SSTX0- |
| C4 | USB_SSRX0+ | USB_SSRX0+ | D4 | USB_SSTX0+ | USB_SSTX0+ |
| C5 | GND | GND | D5 | GND | GND |
| C6 | USB_SSRX1- | USB_SSRX1- | D6 | USB_SSTX1- | USB_SSTX1- |
| C7 | USB_SSRX1+ | USB_SSRX1+ | D7 | USB_SSTX1+ | USB_SSTX1+ |
| C8 | GND | GND | D8 | GND | GND |
| C9 | USB_SSRX2- | USB_SSRX2- | D9 | USB_SSTX2- | USB_SSTX2- |
| C10 | USB_SSRX2+ | USB_SSRX2+ | D10 | USB_SSTX2+ | USB_SSTX2+ |
| C11 | GND (fixed) | GND (fixed) | D11 | GND (fixed) | GND (fixed) |
| C12 | USB_SSRX3- | USB_SSRX3- | D12 | USB_SSTX3- | USB_SSTX3- |
| C13 | USB_SSRX3+ | USB_SSRX3+ | D13 | USB_SSTX3+ | USB_SSTX3+ |
| C14 | GND | GND | D14 | GND | GND |
| C15 | DDI1_PAIR6+ | NC | D15 | DDI1_CTRLCLK_AUX+ | DDI1_CTRLCLK_AUX+ |
| C16 | DDI1_PAIR6- | NC | D16 | DDI1_CTRLDATA_AUX | DDI1_CTRLDATA_AUX |
| C17 | RSVD | NC | D17 | EC_PS2DAT1 | EC_PS2DAT1 |
| C18 | RSVD | NC | D18 | EC_PS2CLK1 | EC_PS2CLK1 |
| C19 | PCIE_RX6+ | PCIE_RX6+ | D19 | PCIE_TX6+ | PCIE_TX6+ |
| C20 | PCIE_RX6- | PCIE_RX6- | D20 | PCIE_TX6- | PCIE_TX6- |
| C21 | GND (fixed) | GND (fixed) | D21 | GND (fixed) | GND (fixed) |
| C22 | PCIE_RX7+ | PCIE_RX7+ | D22 | PCIE_TX7+ | PCIE_TX7+ |
| C23 | PCIE_RX7- | PCIE_RX7- | D23 | PCIE_TX7- | PCIE_TX7- |
| C24 | DDI1_HPD | DDI1_HPD | D24 | EC_PS2DAT0 | EC_PS2DAT0 |
| C25 | DDI1_PAIR4+ | NC | D25 | EC_PS2CLK0 | EC_PS2CLK0 |
| C26 | DDI1_PAIR4- | NC | D26 | DDI1_PAIR0+ | DDI1_PAIR0+ |
| C27 | RSVD | NC | D27 | DDI1_PAIR0- | DDI1_PAIR0- |
| C28 | RSVD | NC | D28 | RSVD | NC |
| C29 | DDI1_PAIR5+ | NC | D29 | DDI1_PAIR1+ | DDI1_PAIR1+ |
| C30 | DDI1_PAIR5- | NC | D30 | DDI1_PAIR1- | DDI1_PAIR1- |
| C31 | GND (fixed) | GND (fixed) | D31 | GND (fixed) | GND (fixed) |
| C32 | DDI2_CTRLCLK_AUX+ | DDI2_CTRLCLK_AUX+ | D32 | DDI1_PAIR2+ | DDI1_PAIR2+ |
| C33 | DDI2_CTRLDATA_AUX- | DDI2_CTRLDATA_AUX- | D33 | DDI1_PAIR2- | DDI1_PAIR2- |
| C34 | DDI2_DDC_AUX_SEL | DDI HDMI/DP功能选择PIN，H=HDMI，L=0 （DP） | D34 | DDI1_DDC_AUX_SEL | DDI HDMI/DP功能选择PIN，H=HDMI，L=0 （DP） |
| C35 | RSVD | NC | D35 | RSVD | NC |
| C36 | DDI3_CTRLCLK_AUX+ | DDI3_CTRLCLK_AUX+ | D36 | DDI1_PAIR3+ | DDI1_PAIR3+ |
| C37 | DDI3_CTRLDATA_AUX- | DDI3_CTRLDATA_AUX- | D37 | DDI1_PAIR3- | DDI1_PAIR3- |
| C38 | DDI3_DDC_AUX_SEL | DDI HDMI/DP功能选择PIN，H=HDMI，L=0 （DP） | D38 | RSVD | NC |
| C39 | DDI3_PAIR0+ | DDI3_PAIR0+ | D39 | DDI2_PAIR0+ | DDI2_PAIR0+ |
| C40 | DDI3_PAIR0- | DDI3_PAIR0- | D40 | DDI2_PAIR0- | DDI2_PAIR0- |
| C41 | GND (fixed) | GND (fixed) | D41 | GND (fixed) | GND (fixed) |
| C42 | DDI3_PAIR1+ | DDI3_PAIR1+ | D42 | DDI2_PAIR1+ | DDI2_PAIR1+ |
| C43 | DDI3_PAIR1- | DDI3_PAIR1- | D43 | DDI2_PAIR1- | DDI2_PAIR1- |
| C44 | DDI3_HPD | DDI3_HPD | D44 | DDI2_HPD | DDI2_HPD |
| C45 | RSVD | NC | D45 | RSVD | NC |
| C46 | DDI3_PAIR2+ | DDI3_PAIR2+ | D46 | DDI2_PAIR2+ | DDI2_PAIR2+ |
| C47 | DDI3_PAIR2- | DDI3_PAIR2- | D47 | DDI2_PAIR2- | DDI2_PAIR2- |
| C48 | RSVD | NC | D48 | RSVD | NC |
| C49 | DDI3_PAIR3+ | DDI3_PAIR3+ | D49 | DDI2_PAIR3+ | DDI2_PAIR3+ |
| C50 | DDI3_PAIR3- | DDI3_PAIR3- | D50 | DDI2_PAIR3- | DDI2_PAIR3- |
| C51 | GND (fixed) | GND (fixed) | D51 | GND (fixed) | GND (fixed) |
| C52 | PEG_RX0+ | PEG_RX0+ | D52 | PEG_TX0+ | PEG_TX0+ |
| C53 | PEG_RX0- | PEG_RX0- | D53 | PEG_TX0- | PEG_TX0- |
| C54 | TYPE0# | NC | D54 | COM_PEG_RSV# | PEG总线Lane 反向功能选择，Input，默认正向，低电平反向 |
| C55 | PEG_RX1+ | PEG_RX1+ | D55 | PEG_TX1+ | PEG_TX1+ |
| C56 | PEG_RX1- | PEG_RX1- | D56 | PEG_TX1- | PEG_TX1- |
| C57 | TYPE1# | NC | D57 | TYPE2# | GND |
| C58 | PEG_RX2+ | PEG_RX2+ | D58 | PEG_TX2+ | PEG_TX2+ |
| C59 | PEG_RX2- | PEG_RX2- | D59 | PEG_TX2- | PEG_TX2- |
| C60 | GND (fixed) | GND (fixed) | D60 | GND (fixed) | GND (fixed) |
| C61 | PEG_RX3+ | PEG_RX3+ | D61 | PEG_TX3+ | PEG_TX3+ |
| C62 | PEG_RX3- | PEG_RX3- | D62 | PEG_TX3- | PEG_TX3- |
| C63 | RSVD | NC | D63 | RSVD | NC |
| C64 | RSVD | NC | D64 | RSVD | NC |
| C65 | PEG_RX4+ | PEG_RX4+ | D65 | PEG_TX4+ | PEG_TX4+ |
| C66 | PEG_RX4- | PEG_RX4- | D66 | PEG_TX4- | PEG_TX4- |
| C67 | RSVD | RSVD | D67 | GND | GND |
| C68 | PEG_RX5+ | PEG_RX5+ | D68 | PEG_TX5+ | PEG_TX5+ |
| C69 | PEG_RX5- | PEG_RX5- | D69 | PEG_TX5- | PEG_TX5- |
| C70 | GND (fixed) | GND (fixed) | D70 | GND (fixed) | GND (fixed) |
| C71 | PEG_RX6+ | PEG_RX6+ | D71 | PEG_TX6+ | PEG_TX6+ |
| C72 | PEG_RX6- | PEG_RX6- | D72 | PEG_TX6- | PEG_TX6- |
| C73 | GND | GND | D73 | GND | GND |
| C74 | PEG_RX7+ | PEG_RX7+ | D74 | PEG_TX7+ | PEG_TX7+ |
| C75 | PEG_RX7- | PEG_RX7- | D75 | PEG_TX7- | PEG_TX7- |
| C76 | GND | GND | D76 | GND | GND |
| C77 | RSVD | NC | D77 | RSVD | NC |
| C78 | PEG_RX8+ | PEG_RX8+ | D78 | PEG_TX8+ | PEG_TX8+ |
| C79 | PEG_RX8- | PEG_RX8- | D79 | PEG_TX8- | PEG_TX8- |
| C80 |  |  | D80 | GND (fixed) | GND (fixed) |
| C81 | PEG_RX9+ | PEG_RX9+ | D81 | PEG_TX9+ | PEG_TX9+ |
| C82 | PEG_RX9- | PEG_RX9- | D82 | PEG_TX9- | PEG_TX9- |
| C83 | RSVD | NC | D83 | RSVD | NC |
| C84 | GND | GND | D84 | GND | GND |
| C85 | PEG_RX10+ | PEG_RX10+ | D85 | PEG_TX10+ | PEG_TX10+ |
| C86 | PEG_RX10- | PEG_RX10- | D86 | PEG_TX10- | PEG_TX10- |
| C87 | GND | GND | D87 | GND | GND |
| C88 | PEG_RX11+ | PEG_RX11+ | D88 | PEG_TX11+ | PEG_TX11+ |
| C89 | PEG_RX11- | PEG_RX11- | D89 | PEG_TX11- | PEG_TX11- |
| C90 | GND (fixed) | GND (fixed) | D90 | GND (fixed) | GND (fixed) |
| C91 | PEG_RX12+ | PEG_RX12+ | D91 | PEG_TX12+ | PEG_TX12+ |
| C92 | PEG_RX12- | PEG_RX12- | D92 | PEG_TX12- | PEG_TX12- |
| C93 | GND | GND | D93 | GND | GND |
| C94 | PEG_RX13+ | PEG_RX13+ | D94 | PEG_TX13+ | PEG_TX13+ |
| C95 | PEG_RX13- | PEG_RX13- | D95 | PEG_TX13- | PEG_TX13- |
| C96 | GND | GND | D96 | GND | GND |
| C97 | RSVD | RSVD | D97 | RSVD | RSVD |
| C98 | PEG_RX14+ | PEG_RX14+ | D98 | PEG_TX14+ | PEG_TX14+ |
| C99 | PEG_RX14- | PEG_RX14- | D99 | PEG_TX14- | PEG_TX14- |
| C100 | GND (fixed) | GND (fixed) | D100 | GND (fixed) | GND (fixed) |
| C101 | PEG_RX15+ | PEG_RX15+ | D101 | PEG_TX15+ | PEG_TX15+ |
| C102 | PEG_RX15- | PEG_RX15- | D102 | PEG_TX15- | PEG_TX15- |
| C103 | GND | GND | D103 | GND | GND |
| C104 | VCC_12V | VCC_12V | D104 | VCC_12V | VCC_12V |
| C105 | VCC_12V | VCC_12V | D105 | VCC_12V | VCC_12V |
| C106 | VCC_12V | VCC_12V | D106 | VCC_12V | VCC_12V |
| C107 | VCC_12V | VCC_12V | D107 | VCC_12V | VCC_12V |
| C108 | VCC_12V | VCC_12V | D108 | VCC_12V | VCC_12V |
| C109 | VCC_12V | VCC_12V | D109 | VCC_12V | VCC_12V |
| C110 | GND (fixed) | GND (fixed) | D110 | GND (fixed) | GND (fixed) |

结构信息

- 核心板连接器确认：

- BTB Socket 220PIN 0.5mm间距 高度=5mm或者高度=8mm SMT receptacle

Sources for the individual 220-pin receptacle are
Tyco Electronics 3-6318490-6
Foxconn QT012206-1031-2H
ept19

or equivalent  0.5 mm pitch Free Height 220 pin 4H Receptacle

Sources for the combined 440-pin receptacle are:

Tyco Electronics  3-1827231-6

Foxconn  QT012206-1041-3H

ept19

or equivalent  0.5mm pitch Free Height 440 pin 4H Receptacle

- 载板连接器确认：

- BTB Socket 220PIN 0.5mm间距 高度=5mm或者高度=8mm SMT plug

①5mm高度BTB 连接器：

推荐Foxconn QT002206-2131-3H    H=5mm

②8mm 高度BTB 连接器：

推荐Foxconn QT002206-4141-3H   H=8mm